Learnings & Reflections

Candy Shuffle Game — ESP32 Candy Dispenser

1. What Went Well

ESP-NOW worked reliably for board-to-board communication with minimal setup and zero wiring between boards.

The shuffle animation (fast→slow→fast→blackout) created genuine suspense and excitement during testing.

MicroPython proved fast to iterate on — code changes could be tested within seconds via Thonny.

Modular code structure (separate functions per feature) made debugging straightforward and isolated.

The active-LOW IR sensor debounce logic worked reliably with no false triggers once correctly implemented.

2. Challenges & How They Were Solved

| Challenge | Learning | Solution |
| --- | --- | --- |
| GPIO12 strapping pin caused repeated flash corruption | ESP32 has boot-critical strapping pins (GPIO0, GPIO2, GPIO12, GPIO15) — never use GPIO12 with PULL_UP | Moved sensor to GPIO13 |
| Active LOW buzzer rang continuously on boot | MicroPython Pin(OUT) defaults to LOW — active LOW devices activate immediately unless explicitly set HIGH | Set buzzer.value(1) right after pin definition |
| Active LOW IR sensors never triggered | With PULL_UP, pin reads HIGH normally and LOW when blocked — opposite to what was coded | Changed check from value()==1 to value()==0 |
| Both ESP32s appeared as same COM port | Both boards used identical CP2102 USB chips — OS assigns same port number | Saved main.py on A to run standalone from power bank |
| Firmware flash failed with hardware connected | esptool uses same SPI pins as VSPI hardware — connected components interfere during flash | Disconnect all hardware before every flash |
| BLE UUID mismatch between app and firmware | MIT App was built with a different NUS UUID variant than standard Nordic spec | Updated ESP32 firmware UUIDs to match MIT App exactly |

3. Key Technical Learnings

PULL_UP vs active LOW:  Sensors wired with PULL_UP resistors read HIGH (1) when idle and LOW (0) when triggered — always check the active state of your sensor before writing detection logic.

ESP32 strapping pins:  GPIO0, GPIO2, GPIO12, GPIO15 control boot behaviour. GPIO12 (MTDI) HIGH at boot = 1.8V flash — always avoid these as inputs with pull-ups.

ESP-NOW:  Operates independently of WiFi association — just need WLAN in STA mode (active but disconnected). Add peer MAC then send. Sub-millisecond latency on local comms.

MicroPython PWM for servos:  SG90 servos on ESP32 use 50Hz PWM with duty values ~40 (closed) to ~100 (open) — not standard microsecond pulse widths like Arduino. Calibrate per servo.

BLE GATT on MicroPython:  Nordic NUS (UART Service) requires exact UUID matching between peripheral and central. Advertisement must include device name in both adv_data and resp_data for MIT App to discover.

4. What I Would Do Differently

Plan GPIO assignments upfront — check the ESP32 datasheet for strapping and boot-sensitive pins before wiring any hardware.

Test each component in isolation before integration — saved significant debugging time once this practice was adopted.

Add try/except around all ESP-NOW send() calls from the start — prevents silent crashes when the peer board is offline.

Use individual servo closed angles from the beginning — all three servos had different physical zero positions requiring per-servo calibration.

Prototype the MIT App connection early — BLE UUIDs and characteristic assignments are easy to get wrong and hard to debug late in the project.

5. Future Improvements

Add a score counter display (OLED or 7-segment) to track wins across multiple rounds.

Implement difficulty levels — shorter timeout or faster shuffle for harder mode.

Re-enable and tune the buzzer for full audio feedback on START, WIN, LOSE, and TIMEOUT.

Build a physical enclosure with labelled slot openings and a mounted spin wheel.

Add a second Luck Mode where the host can pick which slot to fire individually, not just all three.