Project Planning & Management

Candy Shuffle Game — ESP32 Candy Dispenser

1. Project Scope

Two-ESP32 interactive candy dispenser arcade game

Skill Mode (RFID + shuffle + IR detection) and Luck Mode (BLE jackpot)

Physical build with servos, sensors, LEDs, and smartphone app integration

Deliverables: working hardware, MicroPython firmware, MIT App, documentation

2. Project Phases

| Phase | Description | Status |
| --- | --- | --- |
| 1 — Hardware Setup | Assemble ESP32 A (RFID, LEDs, button) and ESP32 B (servos, IR, NeoPixel) | Complete |
| 2 — Component Testing | Test each component independently with test scripts | Complete |
| 3 — ESP32 A Firmware | RFID reading, shuffle animation, indicator LEDs, button logic | Complete |
| 4 — ESP32 B Firmware | Servo control, IR detection, NeoPixel, buzzer, BLE UART | Complete |
| 5 — ESP-NOW Integration | Wireless communication between A and B, message protocol | Complete |
| 6 — BLE / MIT App | Phone app for Luck Mode, BLE connection to ESP32 B | In Progress |
| 7 — Full Game Testing | End-to-end game flow testing, bug fixes, tuning | In Progress |
| 8 — Final Polish | Servo angle tuning, LED colour calibration, enclosure | Pending |

3. Task Breakdown

ESP32 A Tasks

RFID RC522 driver — SPI wiring and tag reading

Colour tag mapping (Yellow / Green / Blue UIDs)

Shuffle animation — fast/slow/fast/blackout phases

Indicator LED blink sequence on colour detect

Button debounce and START event trigger

ESP-NOW setup and message sending

ESP32 B Tasks

Servo PWM calibration (closed/open duty cycles per servo)

IR sensor active-LOW detection with debounce

NeoPixel colour fill, flash, and WIN/LOSE states

BLE UART advertising and GATT service setup

skill_mode() and luck_mode() game logic

ESP-NOW message handling from A

App Tasks

MIT App Inventor BLE scanning and connection

WIN button → WriteStrings to RX characteristic

StringsReceived handler for result feedback

4. Risk Register

| Risk | Impact | Mitigation |
| --- | --- | --- |
| GPIO12 strapping pin | High — corrupts flash on boot | Moved IR sensor 2 to GPIO13 |
| Active LOW buzzer | Medium — buzzes continuously on boot | Set pin HIGH immediately after definition |
| ESP-NOW send() crash | Medium — crashes if peer offline | Wrap in try/except |
| Servo angles vary per unit | Low — dispenser doesn't open fully | Per-servo CLOSED array, tunable OPEN value |
| BLE UUID mismatch | Medium — app can't connect | Matched ESP32 UUIDs to MIT App UUIDs |
| Both ESPs same COM port | Low — can't debug both simultaneously | Save main.py on A, use power bank |

5. Tools & Stack

MicroPython (ESP-IDF based) on ESP32-WROOM-32

Thonny IDE for flashing and serial monitoring

MIT App Inventor for BLE smartphone controller

ESP-NOW for inter-board wireless communication

esptool.py for firmware flashing