# esp32_b_dispenser.py

# ESP32 B -- Dispenser controller

# Handles: servos, IR sensors, NeoPixel, buzzer, BLE (MIT App), ESP-NOW from ESP32 A

import bluetooth

import network

import espnow

from micropython import const

from machine import Pin, PWM

from neopixel import NeoPixel

import time

# -- ESP-NOW setup -------------------------------------------------------------

sta = network.WLAN(network.STA_IF)

sta.active(True)

sta.disconnect()

en = espnow.ESPNow()

en.active(True)

A_MAC = b'\x00\x70\x07\x3A\x6E\xA4'   # ESP32 A MAC address

en.add_peer(A_MAC)

def send_msg(msg):

en.send(A_MAC, msg)

print("ESP-NOW TX: " + msg)

def check_msg():

result = en.recv(0)     # non-blocking

if result[0] is not None:

_, data = result

return data.decode().strip()

return None

# -- Servos --------------------------------------------------------------------

SERVO_CLOSED = [40, 65, 60]  # closed angles per servo (1, 2, 3)

SERVO_OPEN   = 97

DISPENSE_MS  = 800

servos = [PWM(Pin(25), freq=50),

PWM(Pin(26), freq=50),

PWM(Pin(33), freq=50)]

for i, s in enumerate(servos):

s.duty(SERVO_CLOSED[i])

def dispense(idx):

servos[idx].duty(SERVO_OPEN)

time.sleep_ms(DISPENSE_MS)

servos[idx].duty(SERVO_CLOSED[idx])

def dispense_all():

for s in servos:

s.duty(SERVO_OPEN)

time.sleep_ms(DISPENSE_MS)

for i, s in enumerate(servos):

s.duty(SERVO_CLOSED[i])

# -- IR Sensors (active HIGH, PULL_UP) -----------------------------------------

ir = [Pin(4,  Pin.IN, Pin.PULL_UP),

Pin(13, Pin.IN, Pin.PULL_UP),

Pin(14, Pin.IN, Pin.PULL_UP)]

# -- Buzzer (DISABLED) --------------------------------------------------------

buzzer = Pin(18, Pin.OUT)

buzzer.value(1)   # HIGH = off (active LOW buzzer — keep silent)

def beep(times=1, on_ms=120, off_ms=80):

pass  # buzzer disabled

# -- NeoPixel (16 LEDs) --------------------------------------------------------

NUM_LEDS = 16

np = NeoPixel(Pin(5), NUM_LEDS)

def np_fill(r, g, b):

for i in range(NUM_LEDS):

np[i] = (r, g, b)

np.write()

def np_off():

np_fill(0, 0, 0)

def np_flash(r, g, b, times=3, ms=300):

for _ in range(times):

np_fill(r, g, b)

time.sleep_ms(ms)

np_off()

time.sleep_ms(ms)

COLOUR_RGB = {

"YELLOW": (0, 0, 255),    # swapped — was showing wrong colour

"GREEN":  (0, 255, 0),

"BLUE":   (255, 160, 0),  # swapped — was showing wrong colour

}

# -- BLE UART (Nordic NUS -- MIT App Inventor BluetoothLE) --------------------

_IRQ_CONNECT    = const(1)

_IRQ_DISCONNECT = const(2)

_IRQ_WRITE      = const(3)

_UUID_UART = bluetooth.UUID("6E400001-B5B4-4455-8D65-B8843E86D541")

_UUID_TX   = (bluetooth.UUID("6E400003-B5B4-4455-8D65-B8843E86D541"), bluetooth.FLAG_NOTIFY,)

_UUID_RX   = (bluetooth.UUID("6E400002-B5B4-4455-8D65-B8843E86D541"), bluetooth.FLAG_WRITE,)

_UART_SVC  = (_UUID_UART, (_UUID_TX, _UUID_RX,),)

class BLEUART:

def __init__(self, name="CandyESP32"):

self._ble = bluetooth.BLE()

self._ble.active(True)

self._ble.irq(self._irq)

((self._tx, self._rx),) = self._ble.gatts_register_services((_UART_SVC,))

self._connected = False

self._buf = b""

self._name = name

self._advertise()

def _advertise(self):

n = self._name.encode()

adv = b"\x02\x01\x06" + bytes([len(n) + 1, 0x09]) + n

self._ble.gap_advertise(100_000, adv_data=adv, resp_data=adv)

print("BLE advertising as '" + self._name + "'")

def _irq(self, event, data):

if event == _IRQ_CONNECT:

self._connected = True

print("BLE: phone connected")

elif event == _IRQ_DISCONNECT:

self._connected = False

self._buf = b""

print("BLE: phone disconnected - re-advertising")

self._advertise()

elif event == _IRQ_WRITE:

self._buf += self._ble.gatts_read(self._rx)

def readline(self):

if b"\n" in self._buf:

line, self._buf = self._buf.split(b"\n", 1)

return line.decode().strip()

return None

def any(self):

return len(self._buf) > 0

# -- Game logic ----------------------------------------------------------------

WAIT_TIMEOUT_MS = 5000

def skill_mode(colour, winning_slot):

rgb = COLOUR_RGB.get(colour, (255, 255, 255))

np_fill(*rgb)

print("SKILL MODE | colour=" + colour + " | winning slot=" + str(winning_slot + 1))

# Debug: print IR values at start so we can see if any are stuck HIGH

print("IR values at start: " + str([ir[i].value() for i in range(3)]))

print("Waiting for hand... (5 seconds)")

start = time.ticks_ms()

last_print = start

while time.ticks_diff(time.ticks_ms(), start) < WAIT_TIMEOUT_MS:

# Print IR values every second so we can monitor

now = time.ticks_ms()

if time.ticks_diff(now, last_print) >= 1000:

print("IR: " + str([ir[i].value() for i in range(3)]))

last_print = now

for i in range(3):

if ir[i].value() == 0:       # active LOW - 0 means object detected

time.sleep_ms(30)        # debounce

if ir[i].value() == 0:

print("Hand detected at slot " + str(i + 1))

if i == winning_slot:

print("CORRECT SLOT - WIN!")

np_fill(0, 255, 0)

beep(3, 100, 80)

dispense(winning_slot)

time.sleep_ms(2000)

np_off()

send_msg("WIN")

else:

print("WRONG SLOT (slot " + str(i+1) + ") - LOSE")

np_fill(255, 0, 0)

beep(1, 600, 0)

time.sleep_ms(2000)

np_off()

send_msg("LOSE")

return

time.sleep_ms(50)

# Timeout

print("TIMEOUT - no hand detected")

np_flash(255, 80, 0, times=3, ms=300)

beep(2, 200, 150)

np_off()

send_msg("TIMEOUT")

def luck_mode():

print("LUCK MODE - all dispensers firing!")

np_fill(255, 160, 0)

time.sleep_ms(500)

dispense_all()

np_fill(0, 255, 0)

beep(5, 80, 60)

time.sleep_ms(2000)

np_off()

# -- Startup -------------------------------------------------------------------

np_fill(255, 255, 255)   # white = game ready, waiting for first round

beep(2, 80, 80)    # 2 short beeps = ready

ble = BLEUART("CandyGame")

print("=" * 40)

print("  ESP32 B DISPENSER READY")

print("=" * 40)

# -- Main loop -----------------------------------------------------------------

while True:

# ESP-NOW: messages from ESP32 A

msg = check_msg()

if msg:

print("ESP-NOW <- " + msg)

if msg == "START":

# Player pressed START on ESP32 A — single beep

beep(1, 80, 0)

elif msg.startswith("COLOR:"):

# Color detected — light NeoPixel in that colour immediately

colour = msg[6:].strip()

rgb = COLOUR_RGB.get(colour, (255, 255, 255))

np_fill(*rgb)

print("NeoPixel -> " + colour)

elif msg.startswith("SKILL:"):

# Shuffle done — run skill mode with winning slot

try:

body = msg[6:]

colour, slot_s = body.split(",")

slot = int(slot_s) - 1

skill_mode(colour.strip(), slot)

except Exception as e:

print("Parse error: " + str(e))

# BLE: WIN from MIT App (Luck Mode)

if ble.any():

cmd = ble.readline()

if cmd and cmd.strip().upper() == "WIN":

print("BLE <- WIN")

luck_mode()

time.sleep_ms(50)